| Parameter |  | Required @ 3.035 GHz |
| --- | --- | --- |
| Operating frequency |  | 3.035 GHz |
| Number of turns |  | 3 |
| Helix centerline radius |  | 20.94 mm |
| Pitch / vertical spacing |  | 29.27 mm |
| Wire |  | 18 AWG |
| Wire diameter |  | ≈1.024 mm |
| Wire radius |  | ≈0.512 mm |
| Ground-plane radius |  | 56.29 mm |
| Ground-plane diameter |  | 112.59 mm |
| Total axial helix length |  | 87.82 mm |
| Circumference/turn |  | 131.58 mm |
| Slant length/turn |  | 134.79 mm |
| Pitch angle |  | 12.54° |

HELICAL_ANTENNA_MODIFIED_DIMENSTIONS

HELICAL_ANTENNA_MODIFIED_DIMENSTIONS

| Performance Parameter | Typical Optimized Value | Physical Meaning & Impact |
| --- | --- | --- |
| Operating Frequency | 3.035GHz | Centered precisely on the target wireless band or atomic state transition. |
| Reflection Loss(S11) | ≤ (-15 dB to -25 dB) | Excellent impedance match; less than 3% of power bounces back. |
| VSWR | 1.1 : 1 to 1.4 : 1 | Highly efficient transmission line power transfer. |
| Directivity | 10.0 dBi – 14.5 dBi | Tightly concentrates electromagnetic energy into a forward-facing beam. |
| Antenna Gain | 9.5 dB – 14.0 dB | Forward beam power factor accounting for minor metal trace losses. |
| Axial Ratio | < 1.5 dB | High-purity Circular Polarization (CP) essential for selective state driving. |